2020 ACM/IEEE 47th Annual International Symposium on Computer Architecture (ISCA)

Accel-Sim: An Extensible Simulation Framework

for Validated GPU Modeling

| Mahmoud Khairy Purdue University abdallm@purdue.edu | Zhesheng Shen Purdue University shen203@purdue.edu | Tor M. Aamodt University of British Columbia aamodt@ece.ubc.ca | Timothy G. Rogers Purdue University timrogers@purdue.edu |
| --- | --- | --- | --- |
| Abstract—In computer architecture, signiﬁcant innovation fre- quently comes from industry. However, the simulation tools used by industry are often not released for open use, and even when they are, the exact details of industrial designs are not disclosed. As a result, research in the architecture space must ensure that assumptions about contemporary processor design remain true. To help bridge the gap between opaque industrial innovation and public research, we introduce three mechanisms that make it much easier for GPU simulators to keep up with industry. First, we introduce a new GPU simulator frontend that minimizes the effort required to simulate different machine ISAs through trace-driven simulation of NVIDIA’s native machine ISA, while still supporting execution-driven simulation of the virtual ISA. Second, we extensively update GPGPU-Sim’s performance model to increase its level of detail, conﬁgurability and accuracy. Finally, surrounding the new frontend and ﬂexible performance model is an infrastructure that enables quick, detailed validation. A com- prehensive set of microbenchmarks and automated correlation plotting ease the modeling process. We use these three new mechanisms to build Accel-Sim, a detailed simulation framework that decreases cycle error 79 percentage points, over a wide range of 80 workloads, consisting of 1,945 kernel instances. We further demonstrate that Accel-Sim is able to simulate benchmark suites that no other open-source simulator can. In particular, we use Accel-sim to simulate an additional 60 workloads, comprised of 11,440 kernel instances, from the machine learning benchmark suite Deepbench. Deep- bench makes use of closed-source, hand-tuned kernels with no virtual ISA implementation. Using a rigorous counter-by-counter analysis, we validate Accel-Sim against contemporary GPUs. Finally, to highlight the effects of falling behind industry, this paper presents two case-studies that demonstrate how incorrect baseline assumptions can hide new areas of opportunity and lead to potentially incorrect design decisions. Index Terms—GPGPU, Modeling and Simulation I. INTRODUCTION Research cannot look ahead, if its baseline assumptions are too far behind. To keep up with industry, state-of-the- art academic and public research must both be aware of and adapt to changes in contemporary designs. In the computer architecture space, keeping up with proprietary industrial ma- chines is a challenge. This is a problem in all segments of the processor industry, but perhaps a more acute challenge in the programmable accelerator space, where the rapid scaling of parallelism, introduction of new processing pipelines (i.e. Tensor Cores [10]) and undocumented changes to both the microarchitecture and Instruction Set Architecture (ISA) are commonplace in each new product generation. GPU architec- tures have widely embraced the use of a virtual ISA (vISA), |  | which provides hardware vendors with tremendous ﬂexibility to make machine ISA (mISA) changes while still maintaining binary compatibility. Vendors like NVIDIA keep the operation of these ISAs private, while others like AMD document each new machine ISA, but freely make drastic changes which open-source simulators must then implement. This situation presents three separate, but related challenges: (1) How do academic researchers quickly simulate a new, often undocumented ISA every year and a half? (2) Once func- tionally correct, how are changes to the architecture detected and modeled? (3) What is a sustainable, rigorous validation mechanism to ensure that new baselines are still tracking industrial designs? We present a new simulation framework, Accel-Sim, that is designed to address all three challenges. Accel-Sim introduces a ﬂexible frontend, that enables it to operate in either trace- or execution-driven mode. Accel-Sim includes a trace-generation tool (built using the NVBit [70] binary instrumentation tool), that produces machine ISA in- struction traces from any CUDA binary, including those that use closed-source libraries, like cuDNN [43]. These machine ISA traces are then converted into an ISA-independent in- termediate representation that is input to the performance model. The trace-driven frontend allows Accel-Sim to simulate the machine ISA in new cards without implementing the ISA’s functional model and increases the accuracy of the simulator over executing the virtual ISA. However, trace-based simulation has its drawbacks. Evaluating new designs that rely on the data values stored in registers or memory [72] and global synchronization mechanisms [63] are either not possible or very difﬁcult to study without emulation-based execution-driven simulation. Since Accel-Sim is based on an ISA-independent performance model, it is also capable of running emulation-based execution-driven simulations using NVIDIA’s relatively stable, well documented virtual ISA, PTX. Accel-Sim is the ﬁrst academic simulation framework to support contemporary CUDA applications, modern Source and ASSembly (SASS) machine ISAs and simulate hand-tuned assembly in closed-source GPU binaries. Table I presents a survey of the open-source GPU simulation space. Over the last decade, several GPU simulators have been developed, each serving a different purpose. MacSim [28] is an early GPU simulator that performs trace-based simulation of NVIDIA’s virtual ISA and includes a Fermi-like microar- chitecture model. Recent work on MacSim has extended the |  |

978-1-7281-4661-4/20/$31.00 ©2020 IEEE                                             473

DOI 10.1109/ISCA45697.2020.00047

Authorized licensed use limited to: Purdue University. Downloaded on December 01,2021 at 17:51:44 UTC from IEEE Xplore.  Restrictions apply.

TABLE I: Landscape of open-source GPU simulation. Accuracy numbers are taken from each simulator’s respective publication.

|  | GPGPU- Sim 3.x [5] | Gem5- APU [6], [18] | MGPU- Sim [67] | MacSim [14], [28] | Multi2- Sim [15], [69] | Accel-Sim |
| --- | --- | --- | --- | --- | --- | --- |
| ISA | vISA + mISA GT200 | vISA + AMD mISA | vISA + AMD mISA | vISA + Intel mISA | vISA + mISA Kepler + mISA AMD | vISA + NVBit-generated [70] mISA |
| Front-end | Execution | Execution | Execution | Trace | Execution | Trace- and Execution-driven |
| Validated perf. model | Fermi | AMD | AMD | Fermi | Kepler | Kepler, Pascal, Volta, Turing |
| Validated Workloads | 14 | 10 | 7 | N/A | 24 | 80 |
| Reported Accuracy (error %) | 35% [1] | 42% [18] | 5.5% [67] | N/A | 19% [15] | 15% |
| Simulation rate (KIPS) | 3 | N/A | 28 [67] | N/A | 0.8 [67] | 12.5 (Trace-driven mode) 6 (Exec-driven mode) |
| Multi-threaded simulation | X | X | ✓ | X | X | X |
| Hand-tuned NVIDIA libraries (i.e. Volta cuDNN) | X | X | X | X | X | ✓ |

simulator to execute traces for Intel GPUs [14]. Multi2-Sim is a versatile GPU simulator that has emulation-based functional simulation  for  both  the  virtual  ISA  and  a  subset  of  older machine ISAs from both AMD and NVIDIA. GPGPU-Sim [5] is a CUDA-centric simulator capable of functionally executing NVIDIA’s virtual ISA and a subset of an older machine ISA. More recently, Gem5 has been augmented to support an APU performance model for AMD’s virtual and machine ISAs [6]. MGPU-Sim  [67]  is  a  parallel  GPU  simulator  for  AMD’s virtual and machine ISAs. One of the primary drawbacks with all the simulators that support execution-driven machine ISAs is  the  challenge  of keeping  up  with  changes  to  the  mISAs functional  model.  As  a  result,  only  a  limited  subset  of  the instruction  set  (and  hence  applications)  are  ever  fully  sup- ported. This is especially important for supporting optimized libraries  that  often  use  exotic,  hand-tuned  machine  code  to improve performance,  such  as NVIDIA’s cuDNN. Although recent work has augmented GPGPU-Sim [31] to enable virtual ISA execution of these libraries, the functionality only works in pre-Volta  GPUs.  For  Volta  and  Turing,  cuDNN  executes hand-tuned  SASS kernels for which there is no virtual ISA implementation. Accel-Sim’s support for machine ISA traces bypasses all of these issues. In addition, existing GPU simu- lation frameworks lack a systematic methodology to validate and model new architectural designs quickly.

Shifting  the  focus  from  implementing  an  undocumented functional model enables Accel-Sim to focus on the perfor- mance model. We extensively modify GPGPU-Sim’s perfor- mance model (which we release as part of GPGPU-Sim 4.0) to increase its level of detail, conﬁgurability and accuracy. To facilitate rigorous validation, the performance model outputs a  set  of  counters  that  have   1:1  equivalents  with  hardware data  emitted  by  NVIDIA  proﬁlers,  in  addition  to  the  de- tailed  statistics  that  can  only  be  provided  by   simulation. These counter values are then fed into an automated tuning framework that  generates  correlation plots  and modiﬁes the simulator’s  conﬁguration  ﬁles,  making  it  easier  to  create  a validated performance model for a new GPU.

Using  our new  frontend  and  validation  infrastructure,  we perform an extensive modeling effort that builds on GPGPU- Sim’s  [5]  performance model to create  a more  ﬂexible, ex- tensible  GPU  simulator. Through  careful counter-by-counter validation, we expose several changes to contemporary GPU hardware. We demonstrate Accel-Sim’s ﬂexibility and accu-

racy  by  validating  it  against  four  generations  of  NVIDIA GPUs ranging from Kepler to Turing, performing an extensive side-by-side comparison of Accel-Sim and GPGPU-Sim  3.x modeling a Volta V100 [46].

During the course of this analysis we uncover a number of interesting insights into how contemporary hardware works. We utilized every publicly available resource to construct the performance model, capturing the details of what others have either  disclosed  or  discovered  [26],  [34],  [37],  [73].  In  the process  of correlating  the  memory  system  we  discover  and model several undocumented features, such as: details of the streaming, adaptive L1 data cache, sectoring of both the L1 and L2, a sub-warp, sector-centric memory coalescing policy, and an L2 write policy that conserves memory bandwidth in the presence of sub-sector reads.

Finally, this paper performs two  case-studies to highlight new  research  opportunities  on  contemporary  GPUs  and  to demonstrate  pitfalls  that  are  alleviated  by  having  a  simu- lation  infrastructure  that  is  easier  to  validate.  In  particular, we demonstrate that out-of-order memory access scheduling, which  appears  relatively  ineffective  using  an  older  model, yields a signiﬁcant performance improvement when the level of detail in the memory system is increased to more closely match  hardware.  We   also  demonstrate  that  the   L1  cache throughput  bottleneck  present  in  GPUs  no  longer  exists  in contemporary hardware, decreasing the effectiveness of tech- niques that selectively bypass the L1 for throughput reasons.

This paper makes the following contributions:

•   It introduces Accel-Sim, a simulation framework explic- itly  designed  to  make  modeling  and  validating  future GPUs easier. By utilizing a ﬂexible frontend, capable of switching between execution-driven vISA simulation and trace-driven  mISA  simulation,  we  are  able  to  simulate hand-tuned machine code from NVIDIA binaries without giving  up  the  option  to  perform  execution-driven  sim- ulation  when  appropriate. We  demonstrate Accel-Sim’s ﬂexibility by modeling GPUs from Kepler to Turing.

•   It  performs  a  rigorous,  counter-by-counter  comparison of  our  new  performance  model  against  state-of-the-art simulation when modeling an NVIDIA Volta card, reduc- ing error by 79 percentage points. Through this analysis, we uncover and model several previously undocumented architectural features in contemporary GPUs.

474

Detailed

AcceI-sim Tracer SASS traces

correlation Graphs

Accel-Sim-Trace [Correl=0.922 MAE=35.2%]

correlation Guidance

100M

9

8

7

6

5

lsA Def fiIe

4

3

AcceI-sim

Tuner

New HW

Fig. 1: Accel-Sim’s end-to-end ﬂow.

•   It  introduces  a  comprehensive  set  of  validation  infras- tructure that includes: a set of targeted GPU microbench- marks, an automated parameter tuner and an automatic correlation visualizer, useful for  adapting Accel-Sim to model new designs quickly.

•   It performs a set of case-studies to concretely demonstrate the effects of falling behind industry and uses Accel-Sim to suggest future areas of new research for GPUs.

II.  ACCEL-SIM SIMULATION FRAMEWORK

Figure 1 depicts an overview of our new simulation frame- work.  Accel-Sim   is  composed  of:  (1)  a  ﬂexible  frontend that supports execution-driven simulation of NVIDIA’s virtual ISA  and  trace-driven  simulation  of  contemporary  machine ISAs,  (2)  a  ﬂexible  and  detailed  performance  model,  (3)  a correlation generation tool, and (4) a microbenchmarks-based conﬁguration tuner. Together, the four components reduce the effort required to model contemporary and future GPUs.

A.  Flexible frontend

Our  new  frontend  supports  both  vISA  (PTX)  execution- driven  and  mISA  (SASS)  trace-driven  simulation.  In  trace- driven   mode,   mISA   traces   are   converted   into   an   ISA- independent  intermediate representation,  that has  a  1:1  cor- respondence to the  original  SASS instructions. Table II  de- picts  an  example  of  how  SASS  instructions  from  different machine  generations  and  the  virtual  instructions  from  PTX are translated into the ISA-independent representation used by the performance model. The intermediate format is integrated into GPGPU-Sim 4.0 and represents the interface between the frontend and the performance model. The format includes all the  information  necessary  to  perform  timing  simulation,  in particular:  (1)  the  instruction’s  control  ﬂow  (PC  and  active mask),  (2)  the  instruction’s  datapath  information  (registers accessed  and execution unit)  and  (3) memory  addresses for ld/st instructions. In execution-driven mode, the active mask and memory addresses are computed by emulating the PTX instructions, whereas these values are embedded in the trace when executing the machine ISA.

We generate the traces from NVIDIA GPUs using Accel- Sim’s tracer tool that is built on top of NVbit  [70]. We use base+stride  compression  for the memory traces to keep the trace  sizes  within  an  acceptable  range.  When  a  new  SASS

Instruction Cache

Thread Block Scheduler

| Sub-core |  | Sub-core |
| --- | --- | --- |
| Warp | ….. | Warp |
| Scheduler |  | Scheduler |

…..

Register File

Register                      Register

File                               File

Interconnection Network

Exec Units                  Exec Units

EU. 1                               EU. 1

Memory Partition

Memory Partition

L2 Cache

Memory Partition

L2 Cache

EU2                              EU2

.                                                .

.                                                .

…..

.                                                .

L2 Cache

Crossbar

HBM

/GDDR Channel

HBM

/GDDR Channel

HBM

…..

/GDDR Channel

Constant Cache

L1D Cache / Shared

Memory

# Fig. 2: Updated GPGPU-Sim 4.0 performance model.

ISA is released, users provide the frontend with an ISA Def ﬁle that speciﬁes where each instruction should be executed. This is a relatively simple mapping that can be derived from publicly available information on NVIDIA’s machine ISA [42].

By  not  emulating  hundreds  of  scalar  threads  each  cycle, Accel-Sim’s  trace-driven  mode  improves  simulation  speed versus execution-driven mode. Prior work  [18],  [22] demon- strates  that  executing  GPU  vISAs  may  not  be  an  accurate representation of some workloads. The mISA representation of the program includes register allocation and other compiler optimizations, whereas the vISA assumes an inﬁnite register space and has naive instruction scheduling. Further, supporting SASS gives researchers the ability to simulate closed-source, optimized libraries, such as cuDNN [43], which are written in hand-tuned SASS.

B.  Flexible and detailed performance model

To accurately model contemporary GPUs, we make exten- sive modiﬁcations to GPGPU-Sim 3.x’s performance model. This new performance model is released as part of GPGPU- Sim 4.0 and can be used, independent of Accel-Sim, for PTX simulation.  Accel-Sim  utilizes  our  GPGPU-Sim  4.0  perfor- mance model, interfacing with it through the ISA-independent instruction representation.

Figure  2  depicts  an  overview  of the  performance  model. Streaming  multiprocessors  (SMs)  are  composed  of multiple warp schedulers. Each warp scheduler has a dedicated register ﬁle (RF) and its own execution units (EUs). A warp scheduler with  its  dedicated  RF  and  EUs  is  called  a  sub-core   [10], [55]. Sub-cores are isolated, sharing only the instruction cache and the memory  subsystem. The memory coalescing unit is designed to support sub-warp coalescing on every consecutive group  of N  threads.  Our  performance  model  is  capable  of simulating both  separate  and uniﬁed L1D cache  and  shared memory  scratchpad   [45].  Contemporary  GPU  architectures make use of an adaptive cache mechanism, in which the device driver  transparently  conﬁgures  the  shared  memory  capacity and L1D capacity on a per-kernel basis. Using the adaptive cache, if a kernel does not utilize shared memory, all the on- chip storage will be assigned to the L1D cache [50].

lsA-l nd. Format

Simulation QV100 SM Cycles

AcceI-sim Trace- Driven sAss

Frontend

GPGPU-sim

PTX EmuIation

2

10M

(“FADD”, FPU)

(“IADD”, INT)

9

8

7

6

5

4

3

2

1M

9

8

7

6

1M    2        5    10M       2        5   100M Hardware QV100 SM Cycles

……

HW  Def.h fiIe -

#define WARP_SIZE 32

#define SM_NUM 80

AcceI-sim

correIator Simulation

stats

config FiIes

Hardware stats

Sub-core

Warp

Scheduler

Streaming

Multi-

processor

Streaming

Multi-

processor

Streaming

Multi-

processor

Exec Units

EU1

EU2

.

.

.

AcceI-sim uBench suite

GPGPU-sim 4.0

configurabIe GPGPU-sim Perf ModeI

475

TABLE II: Example  demonstrating how mISA instruction traces  and vISA instructions translate into the ISA-independent intermediate representation used by the performance model. In traces, the mapping between  opcode  and execution unit is provided by the ISA def ﬁle in Figure 1. * indicates these values are computed using emulation.

| Execution Mode | Example Instruction | ISA-independent representation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC | Active mask | Reg info: (dsts, srcs) | Exec. unit | Memory addresses, width | Memory scope |
| [Trace] Kepler mISA | LD.E R4, [R6] | 0x78 | 0xFFFF | R4, R6 | Memory unit | 0x2000,..., 4 | global, L1 cached |
| [Trace] Volta mISA | LDG.E.U32.SYS R4, [R6] | 0x32 | 0x00FF | R4, R6 | Memory unit | 0x4000,..., 4 | global, L1 cached |
| [Trace] Pascal mISA | IADD.X R7,R7,R0 | 0x12 | 0x00EF | R7, R0 | INT unit | - | - |
| [Exec] PTX vISA | ld.global.cg.b32 r1, [r0]; | * | * | R1, R0 | Memory unit | * | global, L2 cached |

With the slowing growth of Moore’s law, domain-speciﬁc processing pipelines continue to be introduced in GPUs (i.e. Tensor cores in Volta [46]). To ensure extensibility with this trend, Accel-Sim supports adding simple execution units from the conﬁguration ﬁle, without the need to update the codebase. To add a new specialized unit, the user declares the new unit in the conﬁguration ﬁle and maps the machine ISA op codes that use this unit in the ISA def ﬁle, as described in Figure 1. If  execution-driven  PTX  support  is  required,  then  the  user must also implement the code to emulate the new instruction’s functionality. To determine the latency and throughput of the new  unit,  the  user  speciﬁes  a  sequence  of  instructions  that can be measured by the microbenchmark tuning framework (described in Section II-C). We follow this process to model Volta and Turing’s Tensor Cores.

Our  GPU  cache  model  supports  a  throughput-oriented, banked,  and  sectored  cache  design  [32],  [62].  The  cache  is ﬂexible enough to model GPUs from Kepler through Turing. We  also  model  the  CPU-GPU  memory  copy  engine,  since all DRAM accesses go through the L2, including CPU-GPU memory copies [53]. To reduce unbalanced memory accesses across L2 memory partitions, which we refer to as partition camping  [3],  [33],  we  add  advanced  partition  indexing  that xors the L2 bank bits with randomly  selected bits from the page  row  bits  using  a  Galois-based  irreducible  polynomial (IPOLY)  interleaving  mechanism  [56].  Partition  camping  is a major problem in contemporary GPUs that have 2n  memory partitions, like High Bandwidth Memory (HBM)  [24] which has 8 channels per stack [33], [51]. IPOLY hashing is guaran- teed to be conﬂict-free for all 2n  strides, which are common in GPGPU applications, and also shows reasonable, deterministic performance for other strides [56].

In the memory system, we model new advances in HBM and GDDR6 [24], [51]. This includes the dual-bus interface to launch row and column commands simultaneously, increasing bank parallelism [51], and detailed HBM timing. Further, we implement well-known memory optimization techniques such as advanced xor-based bank indexing and separate read/write buffers, to reduce memory bank conﬂicts and read-write inter- ference [8], [30], [66].

C.  Tuner and targeted microbenchmarks

Accel-Sim  includes  a  microbenchmark  suite  to  pinpoint latency, bandwidth and geometry changes to known hardware structures from one generation to another. When new hardware is released, the user provides a hardware def  header  ﬁle  to

| #define ITERS 32768 #include <HW_def.h> //L1_ARRAY_SIZE is defined in this header based on HW __global__ void l1_lat(uint32_t *startClk, uint32_t *stopClk, uint64_t *data) { // initialize the pointer-chasing array and warm up L1 cache for (uint32_t i=0; i<(L1_ARRAY_SIZE-1); i++) data[i] = (uint64_t)(data + i + 1); |  |  |
| --- | --- | --- |
| uint64_t * ptr0 = data, ptr1 = NULL; // start timing startClk = clock(); // pointer-chasing for ITERS times // use ca modifier to cache the load in L1 for (uint32_t i=0; i<ITERS; ++i) { asm volatile ("ld.global.ca.u64 ptr1, [ptr0];"); ptr0 = ptr1; //swap the register for the next load } // stop timing stopClk = clock(); } | ## SASS |  |
|  |  | LDG.E.64 R12, [R10]; LDG.E.64 R12, [R12]; LDG.E.64 R14, [R12]; LDG.E.64 R14, [R14]; LDG.E.64 R16, [R14]; LDG.E.64 R16, [R16];. |
| l1_lat<<<1,1>>>(startClk , stopClk , data); printf("L1 Latency = %12.4f cycles\n", (float)(stopClk-startClk) / ITERS); |  |  |

## Fig. 3: L1 latency microbenchmark

the  tuner,  as  shown  in  Figure   1.  The  ﬁle  is  used  by  the microbenchamrks  to  help  discover  non-public  conﬁguration parameters.  The  def  ﬁle  enumerates  a  minimal  amount  of information (such as number of SMs, warp size, etc.) that can be derived from publicly available documents [46].

In  total,  we  have   38  microbenchmarks  that  span  from L1/L2/shared memory latency and attained bandwidth, cache write  policy,  cache  conﬁguration,  access/sector  granularity, number  of  cache  banks,  memory  coalescing  policy,  cache streaming  behavior,  execution  unit  latency/throughput  and DRAM latency/bandwidth for  different  data  elements:  ﬂoat, double and 128-bit vector. To illustrate the general conﬁgura- tion of the microbenchmarks, Figure 3 lists the code for our L1  latency  microbenchmark  that  uses  pointer  chasing  [34], [60] to create data dependencies with minimal overhead. The microbenchmark  is  written  such  that  the  kernel’s  execution time  is  dominated  by  the  L1  latency.  After  execution,  the tuner  reads  the  microbenchmark’s  output  and  generates  a conﬁguration ﬁle for the performance model.

For  other  parameters  that  cannot  be  directly  determined by our microbenchmarks (such as warp scheduling, memory scheduling, the L2 cache interleaving granularity and the L2 cache hashing  function), Accel-Sim  simulates  each possible combination  of  these  four  parameters  on  a  set  of  memory

476

bandwidth microbenchmarks (l1-bw, l2-bw, shd-bw and mem- bw).  The  combination  with  the  highest   average  hardware correlation is chosen by the tuner.

D.  Correlator

An  automated  conﬁguration  tuner  does  not  capture  more drastic architectural changes in different generations. Manual effort is required to model such changes. However, the mag- nitude of the effort can be lessened with quickly generated, targeted  information  on  inaccuracy.  Accel-Sim’s  correlation tool  automates the process of generating counter-by-counter correlation data for different architectures. The tool generates graphs  and  data  that  serve  as  correlation  guidance  to  pin- point workloads and metrics where the simulator is not well correlated  to  hardware.  Using  insights  from  the  correlation of various performance counters over different realistic work- loads  and microbenchmarks, performance bugs or misrepre- sentations in the simulator are identiﬁed and corrected.

E.  Simulation rate

Adding detail and ﬂexibility to a performance model often comes at the expense of increased simulation time. Although Accel-Sim  is  not  multithreaded,  we  take  steps  to  improve its speed. With our improvements, Accel-Sim in trace-driven mode  is   able  to  perform   12.5  kilo  warp  instructions  per second, a 4.3 × simulation time improvement over GPGPU- Sim  3.x.  Half  of  our  speed  improvement  comes  from  the fact that trace-driven mode avoids functional execution. The second  half comes  from  a  simulation  optimization  strategy we call simulation-gating, which is a trade-off between event- driven and cycle-driven simulation. GPU simulations involve thousands of in-ﬂight memory requests and hundreds of active threads. Thus, there will always be something to simulate each cycle. Similar to GPGPU-Sim 3.x, the updated performance model is cycle-driven, ticking all the components every cycle. However,  we  observe  that  the  simulator  ticks  many  empty components  and  spends  a  signiﬁcant  amount  of time  doing non-useful  work.  To  overcome  this  issue,  we  keep  a  status state for each component (core, execution unit, cache, dram channel),  then  the  simulator  only  ticks  active  components every  cycle  and  skips  unnecessary  code  and  loop  iterations when  there  is  no  useful  work  in  the  pipeline.  Finally,  in trace-driven mode, the user can set kernel-based checkpointing to  select  the  desired  hotspot  kernels  to  simulate  and  avoid executing long running initialization kernels, which can further improve simulation speed.

III.  WORKLOADS

To validate Accel-Sim and perform a detailed comparative analysis against prior work, we use a wide range of applica- tions from different domains and benchmark suites. With the exception of a handful of applications with trace-size issues, we run all the applications found in Rodinia 3.1 [9], the CUDA SDK  [40],  Parboil   [65],  Polybench   [16],  CUTLASS   [47], our microbenchmark suite, and cutting-edge machine learning workloads  from  the  Deepbench  benchmark  suite  [35].  The

TABLE III: Workloads used in our study. Table VIII lists the hardware cycles for each workload.

| Benchmark Suite | Workloads | Trace Size [Compressed] |
| --- | --- | --- |
| Rodinia 3.1 [9] | b+tree, backprob, bfs, hotspot, srad-v1, sc, nn, needle, dwt2d, lud, lavaMD, kmeans, myocyte | 302 GB [15 GB] |
| CUDA SDK [40] | sp, blk, vec-add, traspose, conv, scan, sorting-net, walsh-transform, histo, mergesort | 18 GB [1.2 GB] |
| Parboil [65] | sad, sgemm, stencil, cutcp, mri-q, histo, spmv, bfs | 250 GB [12 GB] |
| Polybench [16] | 2dconv, 3dconv, 2mm, 3mm, mvt, atax, bicg, gesummv, gemm, syrk | 743 GB [33 GB] |
| Microbenchamrks | l1-lat, l1-bw, shd-lat, shd-bw, l2-lat, l2-bw, mem-lat, mem-bw, maxﬂops | 3 GB [94 MB] |
| CUTLASS [47] | SGEMM with tensor cores WMMA (10 different input) and SGEMM with normal ﬂoating point (10 different input) | 2.5 TB [125 GB] |
| Deepbench [35] | GEMM bench (10 train, 10 infer- ence), CONV bench (10 train, 10 inference), RNN bench (10 train, 10 inference) | 2.6 TB [130 GB] |

sum  total  is  140  workloads  from  the  applications  listed  in Table III. The applications cfd, heartwall, hotspot3D, huffman, leukocyte, srad  v2 from Rodinia, lbm and tpacf from Parboil, corr, cover, fdtd2d and gram-shm from Polybench are omit- ted  since  their  trace  sizes  are  prohibitively  large.  We  leave developing  trace  compression  techniques  and  gradual  trace creation to support these workloads as future work. We note that without traces, Accel-Sim is  still  able to  execute these apps in execution-driven mode. The total disk  size required for all the traces we simulate is 6.2 TB uncompressed (317 GB compressed) for each card generation. Table III lists the trace size for all the benchmark suites. Generating all 6.2 TB worth of traces takes approximately 48 hours using one GPU.	All the workloads are compiled with CUDA  10, using the compute capability of their respective architecture (i.e. sm  70 in the case of Volta). We run the workloads with the input sizes provided by each benchmark suite. Since deep learning performance is highly  sensitive to input  size, we run Deep- bench with multiple inputs, 10 inputs for training and 10 inputs for inference. For the Volta and Turing cards, we execute two versions of the CUTLASS matrix-multiply apps, one that uses tensor cores and one that uses 32-bit ﬂoating point operations.

For  CUTLASS,  the  input  sizes  from  DeepBench’s  matrix multiply  kernels  are  used.  Hardware  counters  are  collected using the nvprof [44] and nsight [49] command-line proﬁlers from CUDA  10. We collect the timing  data on a per-kernel basis by running the applications several times and averaging the results.

IV.  MODELING DIFFERENT GPU GENERATIONS

To demonstrate Accel-Sim’s ﬂexibility, we model four dif- ferent NVIDIA  GPU  generations  (Kepler, Pascal, Volta  and Turing). First, we generate the traces for all the benchmarks using Accel-Sim’s tracing tool on each card1. Second, we pro- vide the ISA def  and HW Def  ﬁles for each card from publicly available documents [42], [46] to the Accel-Sim frontend and

1We use the Volta (sm  70) traces to  simulate Turing (sm  75), as NVBit support for Turing is not expected until Summer 2020.

477

TABLE IV: Accel-Sim modeling properties across four different GPU generations. SPU: Single-Precision Floating Point Unit.

DPU: Double-Precision Unit. SFU: Special Function Unit.

|  | Kepler TITAN [39] | Pascal TITAN X [41] | Volta QV100 [46] | Turing RTX 2060 [48] |
| --- | --- | --- | --- | --- |
| Machine ISA | sm 35 | sm 62 | sm 70 | sm 75 |
| Core Model | shared model | sub core model | sub core model | sub core model |
| Inst Dispatch/Issue | dual-issue | dual-issue | single-issue | single-issue |
| Execution units | SPUs, SFUs | SPUs, SFUs, DPUs | SPUs, SFUs, Tensor Cores | SPUs, SFUs, Tensor Cores |
| L1 Conﬁguration | 128B line, 1 bank , 96-way, 128B/cycle, 42 cycles | 32B sector, 2 banks, 48-way, 64B/cycle, 80 cycles | 32B sector, adaptive, 4 banks, 256-way, 128B/cycle, 28 cycles | 32B sector, 4 banks, 96-way, 28 cycles |
| L2 Conﬁguration | 32B sector, 24 banks, 190 cycles | 32B sector, 24 banks, 238 cycles | 32B sector, 64 banks, 212 cycles, IPOLY hashing | 32B sector, 24 banks, 226 cycles |
| DRAM Conﬁguration | GDDR5 | GDDR5 | HBM | GDDR6 |

TABLE V: Accel-Sim cycle error and correlation factor across four different GPU generations.

|  | Kepler [39] TITAN |  | Pascal [41] TITAN X |  | Volta [46] QV100 |  | Turing [48] RTX 2060 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MAE | Corr. | MAE | Corr. | MAE | Corr. | MAE | Corr. |
| PTX Exec | 28% | 0.99 | 53% | 0.97 | 34% | 0.98 | 32% | 0.99 |
| SASS Trace | 25% | 0.99 | 30% | 0.97 | 15% | 1.00 | 25% | 0.99 |

tuner respectively, as illustrated in Section II. Third, we run our microbenchmark suite and automated tuner on each card to conﬁgure the performance model’s parameters for the card in  question.  Fourth,  using  Accel-Sim’s  correlation  tool,  we generate a set of graphs for performance counters to validate and improve correlation accuracy. Table IV shows the machine ISA version and modeling parameters obtained using Accel- Sim’s tuner to model each GPU generation. These correlation results drive the development of our performance model and enable  us  to  uncover  a  number  of  interesting  insights  into contemporary hardware in different GPU generations. Three signiﬁcant aspects are:

Sub-Warp,  Sectored  Memory  Coalescer:  We  ﬁnd  that  in Pascal, Volta and Turing, both the L1 and L2 caches have 32B sectors  [27],  with  128B  cache  lines.  This  sub-line  fetching behavior  has  a  signiﬁcant  impact  on  irregular  applications running with the L1 cache enabled. To support an access gran- ularity of 32B into the cache, the memory coalescer operates across sub-warps of eight threads, as opposed to grouping 32 threads into a wide 128B access, as other contemporary GPU simulators  do  when  the  L1  cache  is  enabled. We  speculate that the caches have been sectored to: (1) mitigate the memory over-fetching for uncoalesced accesses [57], and (2) reduce the cache tag overhead, especially the high power consumption as- sociated with the large content-address memory [52] structures required for 4 × more tag comparisons if a 32B line is used as opposed to a 128B with 32B sectors. Our experiments suggest that  Kepler does not  support  such  a coalescing mechanism. Accel-Sim’s  ﬂexibility  allows  it  to  model  either  the  legacy Kepler conﬁguration or the Pascal/Volta/Turing conﬁgurations with reasonable accuracy.

Improved L1 throughput: The L1 cache in Volta and Pascal is  a  streaming  cache  [10],  [45],  which  allows  many  cache misses  to  be  in  ﬂight,  regardless  of how  many  cache  lines

are allocated from the uniﬁed scratchpad + L1 (in Volta) or the relatively limited 24kB L1 cache (in Pascal). By running a  microbenchmark,  similar  to  [37],  we  evaluate  miss-status holding  register  [68]  (MSHR)  throughput  and  ﬁnd  that  the number  of MSHR  entries  has  increased  substantially.  Also, we notice that, independent of L1 conﬁgured size, the MSHR throughput is the same, even if more of the on-chip SRAM storage is devoted to shared memory.

Write-Aware   Cache  Design:  Using  Accel-Sim’s  detailed correlation plots, our initial experiments found that hardware conserves memory bandwidth much more than expected when running write-intense GPU workloads. To understand the be- haviour, we developed an L2 write policy microbenchmark to conﬁrm that the L2 is write-back with a write-allocate policy. Interestingly, we also observed additional behaviour on write misses.  Historically,  there  are  two  different  write  allocation policies [27], fetch-on-write and write-validate. Using Accel- Sim, we ﬁnd that the L2 cache in all four generations applies a version of write-validate that handles the varying granularity of GPU read/write requests. L2 reads have a minimum gran- ularity of 32B, but writes can be sub-32B. When a write to a single byte is received, it writes the byte to the sector, sets a corresponding write bit in a byte-level write mask and sets the sector as valid and modiﬁed. When a sector read request is received to a modiﬁed  sector, it ﬁrst checks if the  sector write-mask is complete, i.e. all the bytes have been written to and the line is fully readable. If so, it reads the sector, avoiding a DRAM access. If the sector is not fully written, it generates a read request for this sector and merges it with the modiﬁed bytes. Applying this write policy saves DRAM bandwidth that can be wasted by read-write interference at the DRAM when sub-sector writes are received at the L2 cache.

To demonstrate Accel-Sim’s ability to model different ISAs and hardware generations in both SASS and PTX simulation, Table  V presents  the  Mean  Absolute  Error  (MAE)  and  the Karl Pearson Coefﬁcient of dispersion (Correl) for simulated cycles across four different GPU cards. Correl is the ratio of the standard deviation to the mean and indicates how closely the trends in both the  simulator data and the hardware data match. Table V demonstrates that correlation is > 0.97 in all instances and error is <= 30% for all SASS simulations. The accuracy gained by executing the machine ISA over the virtual ISA can be as much 2 ×, depending on the generation. The one outlier with the greatest error is Pascal in PTX mode. In Pascal,

478

TABLE VI: Volta GPU conﬁguration for GPGPU-Sim 3.x vs Accel-Sim. INT: Integer operation unit.

|  | GPGPU-Sim 3.x | Accel-Sim (SASS + GPGPU-Sim 4.0) |
| --- | --- | --- |
| Front End | Execution-driven PTX 2.0 | Trace-driven SASS 7.0 |
| #SMs | 80 | 80 |
| SM Conﬁguration | Warps per SM = 64, #Schedulers per SM = 4, #Warps per Sched = 16, RF per SM = 64 KB | +Subcore model, warp scheduler isolation |
| #Exec Units | 4 SPUs, 4 SFUs, 4 WMMA Tensors cores [55] | + 4 DPUs, 4 INTs, 8 HMMA Tensor cores |
| Memory Unit | Fermi coalescer (32 thread coalescer) | sub-warp (8 thread) coalescer + Fair memory issue |
| Shared Memory | Programmable-speciﬁed up to 96 KB | Adaptive (up to 96 KB), latency = 20 cycles |
| L1 cache | 32KB, 128B line, 4 sets, write-evict | +32B sector, adaptive cache (up to 128 KB), Streaming cache, banked, latency = 28 cycles |
| L2 cache | 6 MB, 64 banks, 128B line, 32 sets, Write Back, Naive write policy, LRU, latency=100 cycles | +32B sector, subsector write policy, memory copy engine model, +pseudo- random irreducible polynomial L2 hashing with IPOLY(67) |
| Interconnection | 80x64 crossbar, 32B ﬂit |  |
| Memory Model | GDDR5 model, BW=850 GB/sec, latency=100ns | HBM Model, dual-bus interface, Read/Write buffers, advanced bank indexing |

the L1D  and texture cache  are merged  and L1D caching is disabled by default. Without the L1D as a bandwidth ﬁlter, L2 throughput is more important in Pascal. The poor instruction scheduling in PTX results in a 53% error that is reduced to 30% when SASS instruction scheduling is used.

## V.  A DETAILED CORRELATION ANALYSIS

In  this   section,  we  perform  a  quantitative  analysis  to demonstrate how our new performance model can be used to simulate an NVIDIA Quadro V100 [10]. We compare Accel- Sim  against  the  state-of-the-art  NVIDIA  simulation  model from GPGPU-Sim 3.x [1], [5]. The GPGPU-sim 3.x model is a best-effort attempt to model the Volta card using GPGPU- Sim 3.x without the modeling updates described in this paper. Table VI details the conﬁguration parameters used to model the architecture. The left column of the table details the conﬁg- uration of GPGPU-sim 3.x, obtained by scaling resources. The right column lists changes made possible using Accel-Sim and our updated GPGPU-Sim 4.0 performance model. For the sake of brevity, we refer to GPGPU-Sim 3.x as ‘GPGPU-Sim’ and Accel-Sim  with  GPGPU-Sim  4.0  as  ‘Accel-Sim’.  Note  that both  simulators  are  capable  of  executing  NVIDIA’s  Tensor Core  instructions.  GPGPU-sim uses the  PTX-level WMMA tensor  core  instruction.  Based  on  the  analysis  from  Raihan et  al.  [55],  Accel-Sim  models  the  ﬁne-grained,  SASS-level HMMA instruction. Since Accel-Sim is capable of both SASS- trace and PTX-execution driven simulation, we refer to them as Accel-Sim and Accel-Sim [PTX Mode] respectively.

In this section, we demonstrate the accuracy of our more ﬂexible  performance  model  and  the  effect  of  being   able to  simulate  SASS  traces.  The  correlation  ﬁgures  presented throughout this section plot hardware results from the NVIDIA proﬁlers nvprof [44] and nsight-cli [49] on the x-axis and the simulation results on the y-axis. Depending on the  statistic, there are up to 80 datapoints on each graph (80 applications, without the Deepbench workloads). The blue x represents the GPGPU-Sim result and the circles represent Accel-Sim. Each plot also lists the Correl and error for the statistic. We use two different aggregation techniques for error. Intuitively, the Mean Absolute Error (MAE) makes the most sense since it provides a percentage error and is resistant to outliers. However, cal- culating MAE is not possible if the reference counter is zero

TABLE VII: Error and correlation rates of GPGPU-Sim 3.x versus Accel-Sim when modeling an NVIDIA Volta.

| Statistic | Error |  | Correlation |  |
| --- | --- | --- | --- | --- |
|  | GPGPU- Sim 3.x | Accel- Sim | GPGPU- Sim 3.x | Accel- Sim |
| L1 Reqs | NRMSE=3.04 | NRMSE=0.00 | 0.99 | 1.00 |
| L1 Hit Ratio | NRMSE=1.04 | NRMSE=0.76 | 0.69 | 0.87 |
| Occupancy | NRMSE=0.12 | NRMSE=0.13 | 0.99 | 0.99 |
| L2 Reads | NRMSE=2.67 | NRMSE=0.03 | 0.95 | 1.00 |
| L2 Read Hits | NRMSE=3.24 | NRMSE=0.47 | 0.82 | 0.99 |
| DRAM Reads | NRMSE=5.69 | NRMSE=0.92 | 0.89 | 0.96 |
| Instructions | MAE=27% | MAE=1% | 0.87 | 1.00 |
| Execution Cycles | MAE=94% | MAE=15% | 0.87 | 1.00 |

and creates the appearance of massive error when small values deviate.  As  a  result,  we  cannot  use  the  MAE  for  counters other  than  the  number  of  cycles,  instructions  executed  and Instructions Per Cycle (IPC), which will always be non-zero values. For the remaining  counters,  we use the Normalized Root Mean Squared Error (NRMSE). Table VII summarizes the error and correlation co-efﬁcients for GPGPU-Sim versus Accel-Sim, over a number of important performance metrics.

A.  Overall system correlation

Figure  4  details  a  direct  comparison  of  GPGPU-Sim  to Accel-Sim  over  a  set  of  key  simulation  metrics.  Figure  4a plots  the  ﬁnal  number  of cycles  reported  for  each  applica- tion. Rounded to  2  signiﬁcant  digits, Accel-Sim has  a  1.00 correlation  with  hardware,  and  a  15%  mean  absolute  error. In contrast, GPGPU-Sim has a correlation of 0.87 and 94% error.  Generally,  GPGPU-sim’s  values  are  above  the  x=y line,  meaning  the  simulator  is  over-estimating  the  number of  cycles  for  the  workload.  GPGPU-Sim  is  not  modeling the performance enhancements made to the memory system, which Accel-Sim does.

Accel-Sim’s  increased  accuracy  can  be  attributed  to  two factors:  the  machine  ISA  representation  and  the  updated performance model. To understand the effect of the instruction set, Figure 4b plots the number of warp instructions executed. Since GPGPU-Sim operates on PTX, the correlation is rela- tively low and shows a 27% error. In contrast, Accel-Sim has a

479

GPGPU-Sim 3.x [Correl=0.87 MAE=94%]

Accel-Sim [Correl=1.0 MAE=15%]

GPGPU-Sim 3.x [Correl=0.87 MAE=27%]

Accel-Sim [Correl=1.0 MAE=1%]

GPGPU-Sim 3.x [Correl=0.95 NRMSE=2.67] Accel-Sim [Correl=1.0 NRMSE=0.03]

Simulation Wa r p Instructions

1B

5

5

2

S imu lat ion L2 Reads

2 10M

5

1B

100M

S imu lat ion Cycles

5

2

10M

100M

5

2

2

1M

1M

10M

5

5

100k

10k

2 100k 5

2 10k 5

2

1M

5

2

100k

1000

5

2

100

10k

100M

1M

10k

10k          100k          1M          10M

100               10k               1M              100M

Hardware Cycles

### (a) Cycles

Hardware Warp Instructions

### (b) Instructions

Hardware L2 Reads

GPGPU-Sim 3.x [Correl=0.89 NRMSE=5.69]

Accel-Sim [Correl=0.96 NRMSE=0.92]

GPGPU-Sim 3.x [Correl=0.82 NRMSE=3.22]

Accel-Sim [Correl=0.99 NRMSE=0.47]

(c) L2 Reads

GPGPU-Sim 3.x [Correl=0.99 NRMSE=3.04]

Accel-Sim [Correl=1.0 NRMSE=0.00]

100M

10M

1M 100k	10k 1000

100

1B

Simulation L2 Read Hits

Simulation DRAM Reads

S imu lat ion L1D Reads

100M

10M

1M

100k

10k

1000

100

10

1

100M

10M

1M 100k	10k 1000

10

100

1

10k

1             100           10k            1M          100M

100

100M

1M

1

100             10k              1M            100M

Hardware L2 Read Hits

### (d) L2 Read Hits

Hardware DRAM Reads

### (e) DRAM Reads

Hardware L1D Reads

### (f) Post-Coalescer Accesses

GPGPU-Sim 3.x [Correl=0.69 NRMSE=1.04]

GPGPU-Sim 3.x [Correl=0.88 MAE=48%]

Accel-Sim [Correl=0.98 MAE=22%]

Accel-Sim [Correl=0.87 NRMSE=0.76]

100

GPGPU-Sim 3.x [Correl=0.99 NRMSE=0.12] Accel-Sim [Correl=0.99 NRMSE=0.13]

100

Simulation Occupancy

S imu lation L1D H it Rate

200

80

80

S imu lat ion IPC

150

60

60

100

40

40

20

50

20

0

0

0

0             50           100          150          200

0                                   50                                 100

Hardware L1D Hit Rate

### (i) L1 Cache Hit Rate

0                                   50                                 100

#### Hardware IPC

### (g) Machine Warp Instructions Per Cycle

Hardware Occupancy

### (h) Occupancy

Fig. 4: Correlation of key metrics from Accel-Sim versus GPGPU-Sim 3.x over the 80 workloads (comprised of 1945 kernel instances) that can be executed in both the virtual ISA and the machine ISA. These workloads are listed in Table VIII.

480

Accel-Sim [Correl=0.99 MAE=15%]

Accel-Sim [PTX Mode] [Correl=0.98 MAE=34%]

5

2

Accel-Sim  [PTX  Mode]  and  Accel-Sim.  In  many  of  the CUTLASS sgemm and Rodinia workloads, the error is signiﬁ- cantly reduced by moving to SASS-based simulation. In these workloads, capturing the improved instruction  scheduling in SASS is critical to simulator accuracy.

S imu lat ion Cycles

10M

5

2

1M

5

2 100k 5

2

10k

10k           100k            1M            10M

Hardware Cycles

# Fig.  5:  Cycle  correlation  of  Accel-Sim  in  execution-driven PTX mode versus Accel-Sim in trace-driven SASS mode.

Accel-Sim [Correl=0.95 MAE=33%]

In  addition  to  examining  SASS  versus  PTX,  Table  VIII demonstrates some interesting trends across benchmark suites. In  the  CUDA  SDK,  many  of  the  workloads  are  streaming and  sensitive  to  DRAM  bandwidth.  As  such,  our  detailed HBM  model  and  SASS  representation  reduces  the  error  in these  workloads,  providing  relatively  low  error.  In  the  Ro- dinia suite, which includes a number of workloads that have cache-locality  and more  diverse, irregular behaviour, Accel- Sim’s detailed cache model, combined with SASS instruction scheduling helps to reduce the  error.  Although  Accel-Sim’s overall correlation and accuracy are greatly improved, there are still several benchmarks with a high error. Further investigation reveals that correctly modeling the L2 bandwidth is critical in the Rodinia workloads. Our IPOLY indexing helps to alleviate many L2 bank conﬂicts, however it does not exactly match the hardware’s  indexing  function,  as  described  in  Section  V-B.

Simulation Cycles

100M

9

8

7

6

5

4

3

2

The Polybench workloads are cache capacity  sensitive  [59]. GPGPU-sim  employs  a  ﬁxed  capacity  of  32KB  in  the  L1, whereas the adaptive, sectored cache in Accel-Sim allocates the whole  128KB  of Volta’s uniﬁed on-chip memory to the L1D. The 4 × greater cache capacity and more accurate 32B fetch granularity reduces the error in atax, bicg, gesummv, syrk and mvt from 400% to 20% on average.

10M

9

8

7

6

5

4

3

2

2                  5         10M      2                  5        100M

Hardware Cycles

Fig. 6: Accel-Sim cycle correlation when executing deepbench workloads, which cannot be executed by GPGPU-Sim 3.x or Accel-Sim [PTX Mode]. 60 workloads (comprised of 11,440 kernel instances).

1.0 correlation and a 1% error. Accel-Sim’s small error occurs because some applications execute a non-deterministic number of instructions in hardware. Figure 4b demonstrates that PTX generally executes a reasonable number of instructions. How- ever, the ordering of the instructions is signiﬁcantly different in SASS, resulting in better instruction-level parallelism. To quantify the difference between PTX and SASS more clearly, Figure 5 shows the cycle correlation when using Accel-Sim [PTX Mode] versus Accel-Sim. Figure 5 allows us to examine the  effect  of  SASS  versus  PTX  on  the  same  performance model.  On  average,  simulating  SASS  improves  the   mean absolute error by 2 ×, reducing the error from 34% to 15%. To provide additional insight on an app-by-app basis, Table VIII enumerates the per-application  cycle  error for  GPGPU-Sim,

For the CUTLASS sgemm workloads (without Tensor Core instructions)  the  streaming  L1  cache,  advanced  L2  hashing and support for SASS all contribute to reducing the error in Accel-Sim to less than 5%. Interestingly, Accel-Sim’s error for CUTLASS gemm-wmma is higher than sgemm. In  addition, GPGPU-Sim’s error is generally much lower on gemm-wmma than on sgemm. We ﬁnd that the abstract WMMA PTX instruc- tions are a good approximation for the SASS instructions. For some input sizes, the abstract instructions are an even better representation than Accel-Sim’s SASS model for HMMA. In these instance, Accel-Sim  [PTX  Mode] demonstrates nearly zero cycle error, since it beneﬁts from both the memory system updates and the accuracy of the abstract WMMA instructions.

We also plot the attained occupancy (Figure 4h) and IPC (Figure 4g). The occupancy data tracks well for both GPGPU- Sim and Accel-Sim, both models correctly estimate the initial occupancy, deviation from hardware occurs in imbalanced ker- nels where occupancy falls off at different rates. The purpose of correlating IPC is to demonstrate how well the simulator handles different rates of work. In order to avoid negatively biasing  GPGPU-Sim  (which  uses  a  different  instruction  set than  both  Accel-Sim  and  hardware),  we  use  the  machine instruction count as the numerator to calculate the IPC in both simulators. Figure 4g follows from Figure 4a, where Accel- Sim shows better correlation and less error. GPGPU-Sim’s IPC error is less than its cycle error. Most of GPGPU-Sim’s error occurs in long running apps, where the gap in cycles becomes orders of magnitude larger.

481

| Normalized BW | 1.2 1 0.8 0.6 0.4 0.2 0 | GPGPU-sim 3.x Accel-Sim Volta HW Theoretical BW L1 BW L2 BW Mem BW |
| --- | --- | --- |

# Fig. 7: Measured cache and memory bandwidth for Accel-sim, GPGPU-sim 3.x and Volta hardware .

Finally, Figure 6 plots the Deepbench workloads that make use of the closed-source cuDNN and cuBLAS libraries, that include  hand-tuned  SASS  code  for  which  there  is  no  PTX representation.  Only  Accel-Sim  is  able  to  simulate  these workloads. Generally, Accel-Sim is well correlated, although the  overall  error  is  higher   than  for  the   other  workloads. Further inspection reveals that the Deepbench workloads make extensive use of texture memory and local loads, which could require  additional  modeling  to  improve  their  accuracy.  For hardware occupancy, Accel-sim achieves 0.9 correlation and 0.16 NRMSE on the Deepbench workloads. Low occupancy in RNN workloads has been observed by prior work [76] in silicon  GPUs, which we  also  observe in Accel-Sim. Accel- Sim’s support of NVIDIA’s hand-tuned libraries enables new architecture research into the performance of RNNs and other deep learning networks on GPUs, which is not possible in any other simulation framework.

B.  Achieved bandwidth correlation

GPU workloads are highly sensitive to cache and memory bandwidth, making them important factors for modeling GPUs accurately. Signiﬁcant effort has been expended to design con- temporary GPUs to achieve the highest possible bandwidth. To measure how much memory bandwidth can be saturated, we design a set of microbenchmarks to saturate the memory bandwidth of the L1, L2 and main memory. Figure 7 depicts the  cache  and  memory  bandwidth  attained  for  in  the  Volta card, normalized to the theoretical bandwidth. In Volta  [10], [26], the theoretical bandwidth for both simulators have been conﬁgured  to  supply,  at  most,  the  theoretical  bandwidth  of each component.

As shown in Figure 7, GPGPU-Sim is only able to achieve 33%  of  the  theoretical  L1  bandwidth,  whereas  the  Accel- Sim’s streaming and banked L1 cache is able to attain 85%, within  10% error of the attained hardware bandwidth. In L2 cache, thanks to our advanced Galios-based random hashing and accurate interconnect bandwidth, Accel-Sim mitigates L2 partition  camping,  which  has  been  exacerbated  in  contem- porary  GPUs  with  the  introduction  of  HBM  [3],  [33],  and comes within 22% of hardware. We believe that the hashing mechanism used in contemporary GPUs is highly tuned, and eliminating the last 22% error would require extensive reverse- engineering. We leave improving the L2 bandwidth for future

TABLE  VIII:  Cycle  mean  absolute  error  per  workload  in GPGPU-sim 3.x, Accel-Sim [PTX Mode] and Accel-Sim

| Workload | HW cycles | GPGPU- Sim 3.x Error (%) | Accel-Sim [PTX] Error (%) | Accel-Sim Error (%) |
| --- | --- | --- | --- | --- |
| CUDA SDK |  |  |  |  |
| blk | 10214 | -13 | -11 | -7 |
| conv | 327868 | 3 | 30 | -11 |
| fast-walsh-transform | 731574 | 35 | -6 | 4 |
| merge-sort | 52905 | 17 | -15 | 2 |
| scalar-prod | 300811 | -30 | -30 | -32 |
| sorting-networks | 85713 | 10 | -13 | -10 |
| transpose | 97339 | -2 | 12 | -19 |
| vector-add | 140161 | -22 | 32 | -17 |
| scan | 2659445 | 29 | -16 | -16 |
| histo | 61085 | 21 | 21 | -9 |
| CUDA SDK MAE | - | 13 | 16 | 9 |
| Rodinia Suite |  |  |  |  |
| b+tree | 167146 | -7 | -8 | -19 |
| backprop | 63456 | 0 | 4 | -14 |
| bfs | 1107676 | 61 | -11 | -31 |
| dwt2d | 35246 | 90 | 51 | -3 |
| hotspot | 56101 | 63 | 58 | 16 |
| lud | 710043 | 9 | 48 | -9 |
| myocyte | 4561278 | 51 | 42 | 22 |
| neddle | 3016000 | 151 | 43 | -16 |
| srad-v1 | 6031393 | -7 | 42 | 0 |
| sc | 699593 | 82 | -15 | -8 |
| kmeans | 1437879 | 300 | 300 | 63 |
| nn | 7305 | -59 | -40 | -11 |
| lava-md | 5017353 | 20 | 12 | 8 |
| Rodinia MAE | - | 32 | 28 | 8 |
| Parboil Suite |  |  |  |  |
| bfs | 6396950 | -23 | -22 | -47 |
| histo | 5590035 | -56 | -59 | -39 |
| mri-q | 299165 | 220 | 207 | 9 |
| sad | 131272 | 4 | 9 | -47 |
| sgemm | 362333 | 111 | 148 | -16 |
| spmv | 3405773 | 38 | -8 | -7 |
| stencil | 4698755 | -46 | -17 | -10 |
| cutcp | 5321758 | 75 | 75 | 39 |
| Parboil MAE | - | 45 | 37 | 20 |
| Polybench Suite |  |  |  |  |
| 2dconv | 269298 | 62 | 6 | -15 |
| 2mm | 62994676 | 8 | 0 | 0 |
| 3dconv | 1788022 | -26 | -17 | 3 |
| 3mm | 1766299 | 9 | -12 | -12 |
| atax | 3322009 | 432 | -23 | -23 |
| bicg | 3330876 | 604 | -24 | -23 |
| gemm | 587160 | 19 | -12 | -1 |
| gesummv | 2661367 | 1700 | -7 | -7 |
| mvt | 3323425 | 445 | -24 | -23 |
| syrk | 15668564 | 658 | -7 | -7 |
| Polybench MAE | - | 111 | 6 | 4 |
| Microbenchmark Suite |  |  |  |  |
| maxﬂops | 73152 | 39 | 3 | 3 |
| l1-bw | 578838 | 134 | 9 | -4 |
| l1-bw-unroll | 237705 | 304 | 433 | -6 |
| l1-lat | 1177511 | -65 | -2 | -8 |
| shared-bw | 4921085 | -1 | -3 | -3 |
| shared-lat | 70231 | 6 | 12 | 12 |
| l2-bw | 786881 | 202 | 31 | 44 |
| l2-lat | 7184779 | -33 | -18 | -19 |
| mem-bw | 382171 | 21 | 15 | 16 |
| mem-lat | 2389456 | -71 | 10 | -16 |
| Microbenchmark MAE | - | 37 | 14 | 9 |
| CUTLASS GEMM wmma Suite |  |  |  |  |
| wmma-2560x128x2560 | 1455597 | 11 | -1 | 10 |
| wmma-2560x16x2560 | 1325345 | 7 | 0 | 15 |
| wmma-2560x32x2560 | 1338720 | 6 | 0 | 14 |
| wmma-2560x64x2560 | 1376862 | 4 | 0 | 13 |
| wmma-2560x7000x2560 | 19717874 | 12 | 0 | 16 |
| wmma-4096x128x4096 | 2231763 | 32 | 0 | 14 |
| wmma-4096x16x4096 | 2055799 | 10 | 1 | 17 |
| wmma-4096x32x4096 | 2060767 | 12 | 1 | 17 |
| wmma-4096x64x4096 | 2103894 | 17 | 2 | 16 |
| wmma-4096x7000x4096 | 48103472 | 21 | 0 | -4 |
| CUTLASS wmma MAE | - | 11 | 0.5 | 12 |

482

| CUTLASS sgemm Suite |  |  |  |  |
| --- | --- | --- | --- | --- |
| sgemm-2560x1024x2560 | 10525540 | 96 | 54 | -3 |
| sgemm-2560x128x2560 | 5272700 | 97 | 54 | -3 |
| sgemm-2560x16x2560 | 5199087 | 97 | 55 | -2 |
| sgemm-2560x2560x2560 | 26224856 | 95 | 55 | -3 |
| sgemm-2560x32x2560 | 5193979 | 97 | 55 | -2 |
| sgemm-2560x512x2560 | 5289253 | 97 | 54 | -3 |
| sgemm-2560x64x2560 | 5187370 | 98 | 55 | -2 |
| sgemm-4096x128x4096 | 8352408 | 113 | 55 | -3 |
| sgemm-4096x16x4096 | 8266794 | 97 | 55 | -2 |
| sgemm-4096x64x4096 | 8240064 | 100 | 56 | -1 |
| CUTLASS sgemm MAE | - | 98 | 54 | 2 |

work. For memory bandwidth, our detailed HBM model with a dual-bus interface, the random partition indexing and write buffers employed in Accel-Sim achieves 82% of the theoretical memory  bandwidth  and  very  close  to  the  85%  attained  in hardware.  In  the  GPGPU-Sim,  read/write  interference,  low bank-group level parallelism and naive L2 write policy waste memory bandwidth leading it to attain only 62% bandwidth.

C.  L1  Cache Correlation

Figure 4f plots correlation for the post-coalescer accesses received  by  the  L1  cache.  As  Figure  4f  shows,  Accel-Sim has a correlation co-efﬁcient of 1.00 correlation and no error. This is primarily due to the implementation of the  subwarp coalescer and the fact that the L1 cache is sectored. Figure 4i plots the corresponding hit rate for the L1D cache. We note that this statistic is particularly hard to correlate, given that the warp scheduling policy, cache replacement policy and hit/miss accounting decisions in the proﬁler can skew the results. For example, through microbenchmarking, we determined that the hardware  proﬁler  will  count  a  sector  miss  on  a  cache  line whose tag is already present as a hit, but will still send the sector access down to the L2. Effectively, the proﬁler appears to be counting  128B line cache tag hits, even if the sector is missed. Even with these variables, Accel-sim’s cache model achieves only 0.77 NRMSE and a correlation co-efﬁcient of 0.87. In the apps where pervasive error still exists, we believe more  detailed  reverse  engineering  of  the  warp   scheduling policy and replacement policy will help.

GPGPU-sim  have  a  high  correlation,  but  an  NRMSE  of 3.06  in  L1D  read  accesses.  GPGPU-sim  does  not  model  a sectored cache, thus one 128B access will count as four sector accesses in the hardware. Completely divergent applications show  consistent  behaviour  with  hardware  since  32  unique sectors are generated in hardware and 32 unique cache lines are generated in GPGPU-sim.

D.  L2  Cache Correlation

Although all L1D read misses are sent to L2 cache, Accel- Sim’s L2 reads has higher correlation and less error compared to the L1 hit rate. This result conﬁrms our observation that the proﬁler’s L1 hit/miss accounting decisions are different from our assumption. As  shown in Figure 4c, Accel-sim exhibits 1.00 correlation and normalized error reaching only 0.03. We also collected information on write transactions, and noticed similar behaviour.

We attempted to correlate the L2 cache read hit rate with simulation results, but found that the proﬁler gives inconsistent results,  some  of  which  are  greater  than  100%.  So  instead, we correlated with the number of L2 hits, which gave more grounded readings. Figure 4d presents the L2 read hit corre- lation. Overall, Accel-sim achieves 0.99 correlation and 0.47 normalized  error. Again,  at the L2,  scheduling  effects  from the order in which memory accesses reach the L2 will have an effect on it’s hit rate.

E.  DRAM Correlation

Finally,  Figure  4e  plots  the  correlation  of  DRAM  read accesses. The NRMSE is 0.93 and has a 0.96 correlation co- efﬁcient.  The  DRAM  read  correlation  remains  high  and  is more accurate for larger kernels. However, Accel-sim shows a high level of error for smaller workloads.

The  results  in  GPGPU-sim  are  signiﬁcantly  worse  than those in Accel-Sim. Interestingly, the reason for the massive error in the GPGPU-sim is due to its per-cacheline fetch on write  policy.  The  end  result  of  that  policy  was  that  every write to the writeback L2 fetched 4 32-byte memory values from DRAM, which is why the DRAM reads of the GPGPU- sim are consistently overestimated. Accel-sim has eliminated this problem by sectoring the cache and implementing a more accurate write policy, as explained in Section IV.

VI.  CASE STUDIES

This section presents the effect of memory system simula- tion detail on architectural design decisions. A less detailed architecture model may lead to unrealistic issues or incorrect conclusions  that  are  not  relevant  to  state-of-the-art  designs already  being  used  in  industry.  This   section  demonstrates how our more accurate GPU model removes the well-studied bottleneck  of cache  throughput  and  opens  up  new  research opportunities in chip-wide memory issues.

L1  cache throughput: The L1 cache throughput bottleneck for  GPUs  has  been  well  explored  in  literature  using  cache bypassing  and  warp  throttling  techniques   [25],  [29],  [61]. Due to massive multithreading, GPGPU L1 caches can suffer from severe resource contention (e.g. MSHRs and cache line allocation  [25]). However, contemporary GPUs are designed such that the L1 cache is not a throughput bottleneck. Modern GPUs  employ  advanced  architecture  techniques  to  improve L1 cache throughput, such as sectored, banked, adaptive and streaming  L1   caches.  These  techniques   shift  the  memory bottleneck from the L1 cache to the lower levels of the memory hierarchy. Figure 8 plots the L1 cache reservation fails per kilo cycles for cache sensitive workloads for GPGPU-sim vs Accel- Sim. The new L1 model eliminates reservation fails and the L1 cache is no longer a throughput bottleneck. Consequentially, any design aimed at mitigating this L1 throughput bottleneck is less effective on a contemporary GPU, which is reﬂected in our new performance model.

483

MSHR_RESERV_FAIL       LINE_ALLOC_FAIL       QUEUE_FULL

1000

800

600

400

200

0

srad-v1         srad-v2              sc                 mm               mvt          gesummv          atax

# Fig.  8: L1  cache reservation  fails per kilo  cycles  for  cache sensitive workloads in both Accel-Sim and GPGPU-sim 3.x.

GPGPU-sim 3.x           Accel-sim

6

5

4

3

2

1

0

Fig.  9:  FR  FCFS  performance  normalized  to  the  FCFS  in Accel-Sim and GPGPU-sim 3.x for memory intensive work- loads in Volta.

Memory      scheduling      sensitivity:     Throughput-oriented GPGPU workloads are often sensitive to memory bandwidth. Thus,   the   DRAM   memory    access   scheduler    plays   an important  role  to  efﬁciently  utilize  bandwidth  and  improve the  performance  of  memory-intensive  workloads  (i.e.  those that utilize > 50% of DRAM bandwidth). Figure 9 shows the sensitivity  of memory-intensive  GPGPU  workloads  for  two memory  scheduling  policies:  the  na…ıve  ﬁrst-come  ﬁrst-serve scheduling   (FCFS)   and   the   advanced   out-of-order   First- row-ready ﬁrst-come ﬁrst-serve scheduling (FR  FCFS)  [58]. FR  FCFS   performance   is   normalized   to   FCFS   in   both Accel-Sim and GPGPU-sim to demonstrate the performance improvement from more advanced scheduling. In the GPGPU- Sim, some workloads are insensitive or show little difference between  the  two  scheduling  policies.  On  average,  applying FR  FCFS in GPGPU-sim increases performance by 20%.

In Accel-Sim, the memory scheduling policy is more criti- cal. Applying FR  FCFS to Accel-Sim improves performance by 2.5 × on average. More detailed coalescing rules, improved on-chip cache throughput, advanced L2 cache interleaving and write allocation policies increase the likelihood of losing page locality,  which  must be recaptured by  out-of-order  memory access  scheduling. Further, with the new features and capa- bilities of the HBM such as dual-bus interface [51], pseudo- independent accesses [23] and per-bank refresh command [24], memory  scheduling  and  the  interaction  with  the  L2  cache indexing will become a more critical issue to investigate. This experiment demonstrates how accurately modeling contempo-

rary GPU hardware reveals performance issues obscured using a less detailed simulation. We believe Accel-Sim opens up a rich new  design  space in  system-level  GPU research where core-level memory issues are alleviated and there is increased sensitivity to chip-wide memory issues.

VII.  RELATED WORK

As Table I illustrates, there has been extensive work done in  GPU   simulation  over  the  last  decade.  Xun  et  al.   [15] model  GPU  Kepler  architecture  [38]  that  is  integrated  with the Multi2sim simulation infrastructure  [69]. The new simu- lator was validated using applications from the CUDA SDK. Power  et  al.  [54]  propose  the  Gem5-GPU  simulator  which models detailed CPU-GPU interaction. Beckmann et al.  [6], [18]  introduce  a  Gem5  GPU  model,  that  simulates  AMD’s GCN architecture. Barrio et al. [12] propose ATTILA, which is  a  trace-driven  GPU  simulator  that  models  the  graphics pipeline for the GT200 architecture. Gubran et al. [17] update GPGPU-sim to  execute  graphics  workloads.  Sniper  [7]  is  a multithreaded  CPU  simulator  that  can  execute  in  trace-  or execution-driven mode using PIN-based callbacks to simulate instructions without emulation. In this vast landscape, Accel- Sim is the only open-source framework, explicitly designed for validation, that can simulate NVIDIA SASS instructions and accurately model contemporary NVIDIA GPUs.

Stephenson   et   al.   [64]   introduce   SASSI,   a   low-level assembly-language instrumentation tool for GPUs. Many pre- vious works  [4],  [20],  [21],  [71],  [74],  [75]  aim  to  estimate the execution time of GPGPU applications throughout source- level analysis, however they do not perform detailed perfor- mance modeling for architecture research. Nowatzki et al. [36] show  that  GPGPU-Sim  is  inaccurate  in  modeling  some  in- core parameters and connection delays. Hongwen et al.  [11] argue  that  GPGPU-Sim  has  a  very  weak  L1  cache  model and  they   suggest  enhancements  to  improve  the  L1  cache throughput. Jain et al. [22] demonstrate a large gap in GPGPU- Sim  3.x’s  memory  system  when  correlating  against  Pascal hardware. Prior work [2], [13], [19] propose a methodology for validating CPU simulation models against real hardware. Nu- merous works have done micro-benchmarking to characterize aspects of GPU architecture for Tesla GT200 [73], Fermi [37], Maxwell  [34], and recently Volta  [26]. They demystiﬁed the L1/L2 cache’s associativity and replacement policy. However, none of these works have investigated the L1 streaming cache, sectoring  of  the  L1/L2  caches   and  the  interaction  of  the memory coalescer with caches or the cache write policy.

VIII.  CONCLUSION

This paper introduces three innovative mechanisms to help solve  the  problem  of  keeping   simulators  up-to-date  with contemporary  designs.  We  propose   a  novel,   ambidextrous GPU frontend that translates both functionally executed vir- tual instructions and machine instruction traces into an ISA- independent format for simulation. Using the ﬂexible frontend, a  detailed  performance  model  is  developed,  which  has  the ability to model contemporary GPUs and execute proprietary

L1 Fails Per Kilo Cycles

GPGPU-sim 3.x

Accel-sim

GPGPU-sim 3.x

Accel-sim

GPGPU-sim 3.x

Accel-sim

GPGPU-sim 3.x

Accel-sim

GPGPU-sim 3.x

Accel-sim

GPGPU-sim 3.x

Accel-sim

GPGPU-sim 3.x

Accel-sim

Normalized Performance

484

binaries that no other open-source simulator can. We believe that Accel-Sim is the most extensively validated open-source GPU  simulation  framework  to   date.  We   demonstrate  that Accel-Sim decreases cycle error from 94% in state-of-the-art simulation to 15%, on a comprehensive suite of workloads.

Accel-Sim’s ISA-independent performance model opens up opportunities to simulate cards from a wide variety of vendors. Work is currently underway to generate AMD GCN3 traces from  Gem5-APU  [6]  such  that  Accel-Sim  can  be  used  to model and validate AMD GPUs. We believe that Accel-Sim will reduce the accuracy gap between industrial and academic simulators on an ongoing basis, increasing the potential impact of accelerator research.

ACKNOWLEDGMENTS

We thank our anonymous reviewers for feedback on early versions  of this paper,  David  Nellans,  Oreste  Villa  and  the NVBit  [70]  developers for early access to the tool and bug ﬁxes  that  facilitated  the  creation  of  the  Accel-Sim   SASS Tracer, and Akshay Jain and Syed Farooq for their help in the early phases of the project. We would also like to recognize all the researchers who have made contributions to GPGPU- Sim since the last major release. This work was supported, in part,  by  NSF  CCF  #1910924  and  the  Applications  Driving Architectures  (ADA)  Research  Center,  a  JUMP  Center  co- sponsored by SRC and DARPA.

REFERENCES

[1]  T. M. Aamodt, W. W. Fung, I. Singh, A. El-Shaﬁey, J. Kwa, T. Hether- ington, A. Gubran, A. Boktor, T. Rogers, and A. Bakhoda, “GPGPU-Sim 3. x manual,” http://gpgpu-sim.org/manual/index.php/Main  Page, 2016.

[2]  A. Adileh, C. Gonzlez-Alvarez, J. M. D. H. Ruiz, and L. Eeckhout, “Racing to Hardware-Validated Simulation,” in IEEE International Sym- posium  on  Performance Analysis  of  Systems  and  Software  (ISPASS), 2019, pp. 58–67.

[3]  A. M. Aji, M. Daga, and W.-c. Feng, “Bounding the Effect of Partition Camping in GPU Kernels,” in  Proceedings of the 8th ACM International Conference on Computing Frontiers (CF), 2011, p. 27.

[4]  N. Ardalani, C. Lestourgeon, K. Sankaralingam, and X. Zhu, “Cross- Architecture  Performance  Prediction  (XAPP)  Using  CPU  Code  to Predict  GPU  Performance,”  in  Proceedings  of  the  48th  International Symposium on Microarchitecture (MICRO), 2015, pp. 725–737.

[5]  A. Bakhoda, G. L. Yuan, W. W. Fung, H. Wong, and T. M. Aamodt, “Analyzing  CUDA  Workloads  Using  a  Detailed  GPU  Simulator,”  in IEEE International Symposium on Performance Analysis of Systems and Software (ISPASS), 2009, pp. 163–174.

[6]  B. Beckmann and A. Gutierrez, “The AMD gem5 APU Simulator: Mod- eling Heterogeneous Systems in gem5,” in  Tutorial at the International Symposium on Microarchitecture (MICRO), 2015.

[7]  T.  E.  Carlson,  W.  Heirman,  and  L.  Eeckhout,  “Sniper:  Exploring the  Level  of  Abstraction  for  Scalable  and  Accurate  Parallel  Multi- Core Simulation,” in  Proceedings of International Conference for High Performance Computing, Networking, Storage and Analysis (SC), 2011,

pp. 1–12.

[8]  N. Chatterjee, M. O’Connor, G. H. Loh, N. Jayasena, and R. Balasub- ramonian, “Managing DRAM Latency Divergence in Irregular GPGPU Applications,” in  Proceedings of the International Conference for High Performance Computing, Networking, Storage and Analysis (SC), 2014, pp. 128–139.

[9]  S. Che, M. Boyer, J. Meng, D. Tarjan, J. W. Sheaffer, S.-H. Lee, and K.  Skadron,  “Rodinia:  A  Benchmark  Suite  for  Heterogeneous  Com- puting,” in  Proceedings of IEEE international symposium on workload characterization (IISWC), 2009, pp. 44–54.

[10]  J. Choquette, O. Giroux, and D. Foley, “Volta: Performance and Pro- grammability,” IEEE Micro 2018, vol. 38, no. 2, pp. 42–52.

[11]  H.  Dai,  C.  Li,  Z.  Lin,   and  H.  Zhou,   “The  Demand  for   a   Sound Baseline in  GPU Memory Architecture Research,” in Proceedings  of the Workshop on Duplicating, Deconstructing and Debunking (WDDD), 2017.

[12]  V. M. Del Barrio, C. Gonzlez, J. Roca, A. Fernndez, and E. Espasa, “ATTILA: A Cycle-Level Execution-Driven Simulator for Modern GPU Architectures,” in  IEEE International Symposium on Performance Anal- ysis of Systems and Software (ISPASS), 2006, pp. 231–241.

[13]  R. Desikan, D. Burger, and S. W. Keckler, “Measuring Experimental Error in Microprocessor Simulation,” in Proceedings of the 28th annual International Symposium on Computer Architecture (ISCA), 2001, pp. 266–277.

[14]  P.  Gera,  H.  Kim,  H.  Kim,  S.  Hong,  V.  George,  and  C.-K.  C.  Luk, “Performance Characterisation and Simulation of Intel’s Integrated GPU Architecture,” in  IEEE International Symposium on Performance Anal- ysis of Systems and Software (ISPASS), 2018, pp. 139–148.

[15]  X.  Gong,  R.  Ubal,   and  D.   Kaeli,  “Multi2Sim  Kepler:  A  Detailed Architectural  GPU  Simulator,”  in   IEEE  International  Symposium  on Performance Analysis of Systems and Software (ISPASS), 2017, pp. 269– 278.

[16]  S. Grauer-Gray, L. Xu, R. Searles, S. Ayalasomayajula, and J. Cavazos, “Auto-tuning  a  High-Level  Language  Targeted  to   GPU  Codes,”  in Innovative Parallel Computing (InPar), 2012, pp. 1–10.

[17]  A. A.  Gubran  and  T. M. Aamodt,  “Emerald:  Graphics  Modeling  for SoC Systems,” in  ACM/IEEE 46th Annual International Symposium on Computer Architecture (ISCA), 2019, pp. 169–182.

[18]  A. Gutierrez, B. M. Beckmann, A. Dutu, J. Gross, M. LeBeane, J. Kala- matianos, O. Kayiran, M. Poremba, B. Potter, S. Puthoor, M. Sinclair, M.  Wyse,  J.  Yin,  X.  Zhang,  A.  Jain,  and  T.  G.  Rogers,  “Lost  in Abstraction: Pitfalls of Analyzing GPUs at the Intermediate Language Level,” in  IEEE International Symposium on High Performance Com- puter Architecture (HPCA), 2018, pp. 608–619.

[19]  A.  Gutierrez, J.  Pusdesris, R.  G.  Dreslinski,  T. Mudge,  C.  Sudanthi, C. D. Emmons, M. Hayenga, and N. Paver, “Sources of Error in Full- System Simulation,” in  IEEE International Symposium on Performance Analysis of Systems and Software (ISPASS), 2014, pp. 13–22.

[20]  S.  Hong  and  H.  Kim,  “An  Integrated  GPU  Power  and  Performance Model,” in  Proceedings of the 37th annual international symposium on computer architecture (ISCA), 2010, pp. 280–289.

[21]  J.-C.  Huang,  J.  H.  Lee,  H.  Kim,  and  H.-H.  S.  Lee,  “GPUMech: GPU Performance Modeling Technique based on Interval Analysis,” in Proceedings of the 47th Annual IEEE/ACM International Symposium on Microarchitecture (MICRO), 2014, pp. 268–279.

[22]  A. Jain, M. Khairy, and T. G. Rogers, “A Quantitative Evaluation of Contemporary GPU Simulation Methodology,” Proceedings of the ACM on Measurement and Analysis of Computing Systems (SIGMETRICS), vol. 2, no. 2, p. 35, 2018.

[23]  JEDEC Standard, “GDDR5X,” JESD232A, 2016.

[24]  ——, “High bandwidth memory (HBM) DRAM,” JESD235, 2020.

[25]  W. Jia, K. A. Shaw, and M. Martonosi, “MRPB: Memory Request Prior- itization for Massively Parallel Processors,” in IEEE 20th International Symposium on High Performance Computer Architecture (HPCA), 2014, pp. 272–283.

[26]  Z.  Jia,  M.  Maggioni,  B.  Staiger,  and  D.  P.  Scarpazza,  “Dissecting the NVIDIA Volta GPU Architecture via Microbenchmarking,” arXiv preprint arXiv:1804.06826, 2018.

[27]  N. P. Jouppi, “Cache Write Policies and Performance,” in 20th Annual International Symposium on Computer Architecture (ISCA),  1993, pp. 191–201.

[28]  H. Kim, J. Lee, N. B. Lakshminarayana, J. Sim, J. Lim, and T. Pho, “MacSim:  A  CPU-GPU  Heterogeneous  Simulation  Framework  User Guide,”  Technical Report, Georgia Institute of Technology, 2012.

[29]  G.  Koo,  Y.  Oh,  W.  W.   Ro,  and  M.  Annavaram,  “Access  Pattern- Aware Cache Management for Improving Data Utilization in GPU,” in Proceedings of the 44th Annual International Symposium on Computer Architecture (ISCA), 2017, pp. 307–319.

[30]  C.  J.  Lee,  V.  Narasiman,  E.  Ebrahimi,   O.  Mutlu,   and  Y.  N.  Patt, “DRAM-Aware Last-Level Cache Writeback: Reducing Write-Caused Interference in Memory Systems,”  Technical Report,  The University of Texas at Austin, TR-HPS-2010-002, 2010.

[31]  J. Lew, D. A. Shah, S. Pati, S. Cattell, M. Zhang, A. Sandhupatla, C. Ng, N. Goli, M. D. Sinclair, T. G. Rogers, and T. M. Aamodt, “Analyzing Machine  Learning  Workloads  Using  a  Detailed  GPU  Simulator,”  in

485

IEEE International Symposium on Performance Analysis of Systems and Software (ISPASS), 2019, pp. 151–152.

[32]  J. S. Liptay, “Structural Aspects of the System/360 Model 85, II: The Cache,” IBM Systems Journal, vol. 7, no. 1, pp. 15–21, 1968.

[33]  Y. Liu, X. Zhao, M. Jahre, Z. Wang, X. Wang, Y. Luo, and L. Eeckhout, “Get Out of the Valley: Power-Efﬁcient Address Mapping for GPUs,” in   ACM/IEEE  45th  Annual  International   Symposium   on  Computer Architecture (ISCA), 2018, pp. 166–179.

[34]  X.  Mei  and  X.  Chu,  “Dissecting  GPU  Memory  Hierarchy  through Microbenchmarking,”  IEEE  Transactions  on  Parallel  and Distributed Systems (TPDS), vol. 28, no. 1, pp. 72–86, 2017.

[35]  S. Narang and G. Diamos, “Baidu Deepbench,” https://svail.github.io/ DeepBench, 2016.

[36]  T. Nowatzki, J. Menon, C.-H. Ho, and K. Sankaralingam, “Architectural Simulators Considered Harmful,” IEEE Micro, vol. 35, no. 6, pp. 4–12, 2015.

[37]  C. Nugteren, G.-J. van den Braak, H. Corporaal, and H. Bal, “A Detailed GPU Cache Model Based on Reuse Distance Theory,” in IEEE 20th

International Symposium on High Performance Computer Architecture (HPCA), 2014, pp. 37–48.

[38]  NVIDIA,  “Kepler  Architecture,”  https://www.nvidia.com/content/PDF/ kepler/NVIDIA-Kepler-GK110-Architecture-Whitepaper.pdf, 2012.

[39]  ——,    “GeForce    GTX    Titan,”    https://www.geforce.com/hardware/ desktop-gpus/geforce-gtx-titan/speciﬁcations, 2013.

[40]  ——, “CUDA C/C++ SDK Code Samples,” http://developer.nvidia.com/ cuda-cc-sdk-code-samples, 2015.

[41]  ——,   “NVIDIA   TITAN   X,”   https://www.nvidia.com/en-us/geforce/ products/10series/titan-x-pascal/, 2016.

[42]  ——,     “CUDA      Binary     Utilities,”      https://docs.nvidia.com/cuda/ cuda-binary-utilities/index.html, 2017.

[43]  ——, “CuDNN Devloper Guide,” https://docs.nvidia.com/deeplearning/ sdk/cudnn-developer-guide/index.html, 2017.

[44]  ——,       “Nvidia        Proﬁler        8.0,”        http://docs.nvidia.com/cuda/ proﬁler-users-guide/index.html, 2017.

[45]  ——,    “Volta    Cache,”    http://on-demand.gputechconf.com/gtc/2017/ presentation/s7798-luke-durant-inside-volta.pdf, 2017.

[46]  ——,   “Volta   V100  White  paper,”  http://images.nvidia.com/content/ volta-architecture/pdf/volta-architecture-whitepaper.pdf, 2017.

[47]  ——, “CUTLASS: CUDA template library for dense linear algebra at all levels and scales,” https://github.com/NVIDIA/cutlass, 2018.

[48]  ——,   “GeForce   RTX   2060,”   https://www.nvidia.com/en-us/geforce/ graphics-cards/rtx-2060/, 2019.

[49]  ——,  “NVIDIA  Nsight  CLI,”  https://docs.nvidia.com/nsight-compute/ NsightComputeCli/index.html, 2019.

[50]  ——,        “Programming         Guide,”        https://docs.nvidia.com/cuda/ cuda-c-programming-guide/index.html, 2019.

[51]  M.  O’Connor,  “Highlights  of  the  High-Bandwidth  Memory  (HBM) Standard,”  in  Memory  Forum  Workshop,  Co-located  with  the  41st

International Symposium on Computer Architecture (ISCA), 2014.

[52]  K.   Pagiamtzis   and    A.   Sheikholeslami,    “A   Low-Power   Content- Addressable  Memory  (CAM)  Using  Pipelined  Hierarchical  Search Scheme,” IEEE Journal of Solid-State Circuits, vol. 39, no. 9, pp. 1512– 1519, 2004.

[53]  Paulius          Micikevicius,           “Kepler          Architecture,”          http: //on-demand.gputechconf.com/gtc/2013/presentations/

S3466-Programming-Guidelines-GPU-Architecture.pdf, 2013.

[54]  J.  Power,  J.  Hestness,  M.  S.  Orr,  M.  D.  Hill,  and  D.  A.  Wood, “gem5-gpu:  A  heterogeneous  CPU-GPU  Simulator,”  IEEE  Computer Architecture Letters (CAL), vol. 14, no. 1, pp. 34–36, 2014.

[55]  M. A. Raihan, N. Goli, and T. M. Aamodt, “Modeling Deep Learning Accelerator  Enabled  GPUs,”  in   IEEE  International   Symposium  on

Performance Analysis of Systems and Software (ISPASS), 2019, pp. 79– 92.

[56]  B.  R.  Rau,  “Pseudo-Randomly  Interleaved  Memory,”  in  Proceedings

of the 18th Annual International Symposium on Computer Architecture (ISCA), 1991, pp. 74–83.

[57]  M. Rhu, M. Sullivan, J. Leng, and M. Erez, “A Locality-Aware Memory Hierarchy for Energy-Efcient GPU Architectures,” in Proceedings of the 46th Annual IEEE/ACM International Symposium on Microarchitecture (MICRO), 2013, pp. 86–98.

[58]  S.  Rixner,  W.  J.  Dally,  U.  J.  Kapasi,  P.  Mattson,  and  J.  D.  Owens, “Memory  Access  Scheduling,”  in   Proceedings  of  the  27th  Annual International Symposium on Computer Architecture (ISCA), 2000, pp. 128–138.

[59]  T.  G.  Rogers,  M.  O’Connor,  and  T.  M.  Aamodt,  “Cache-Conscious Wavefront Scheduling,” in  Proceedings of the 45th Annual IEEE/ACM International Symposium on Microarchitecture (MICRO), 2012, pp. 72– 83.

[60]  R. H. Saavedra-Barrera, “CPU Performance Evaluation and Execution Time  Prediction  Using  Narrow  Spectrum  Benchmarking,”   Technical Report No. UCB/CSD-92-684, University of California, Berkeley,  1993.

[61]  A.  Sethia,  D.  A.  Jamshidi,   and  S.  Mahlke,   “Mascar:  Speeding  up GPU Warps by Reducing Memory Pitstops,” in IEEE 21st International Symposium on High Performance Computer Architecture (HPCA), 2015, pp. 174–185.

[62]  A.  Seznec,  “Decoupled  Sectored  Caches:  conciliating  low tag imple- mentation cost and low miss ratio,” in Proceedings of the 21st Annual International Symposium on Computer Architecture (ISCA),  1994, pp. 384–393.

[63]  M.  D.  Sinclair,  J.  Alsop,  and  S.  V.  Adve,  “Chasing  Away  RAts: Semantics and Evaluation for Relaxed Atomics on Heterogeneous Sys- tems,” inACM/IEEE 44th Annual International Symposium on Computer Architecture (ISCA), 2017, pp. 161–174.

[64]  M.  Stephenson,  S.  K.  S.  Hari,  Y.  Lee,  E.  Ebrahimi,  D.  R.  Johnson, D. Nellans, M. O’Connor, and S. W. Keckler, “Flexible Software Pro- ﬁling of GPU Architectures,” in ACM/IEEE 42nd Annual International Symposium on Computer Architecture (ISCA), 2015, pp. 185–197.

[65]  J.  A.  Stratton,  C.  Rodrigues,  I.-J.  Sung,  N.  Obeid,  L.-W.  Chang, N. Anssari, G. D. Liu, and W. Hwu, “Parboil: A Revised Benchmark Suite for Scientiﬁc and Commercial Throughput Computing,” Center for Reliable and High-Performance Computing, vol. 127, 2012.

[66]  J.  Stuecheli,  D.  Kaseridis,  D.  Daly,  H.  C.  Hunter,  and  L.  K.  John, “The Virtual Write Queue: Coordinating DRAM and Last-Level Cache Policies,” in  Proceedings of the 47th Annual International Symposium on Computer Architecture (ISCA), 2010, pp. 72–82.

[67]  Y. Sun, T. Baruah, S. A. Mojumder, S. Dong, X. Gong, S. Treadway, Y. Bao, S. Hance, C. McCardwell, V. Zhao, H. Barclay, A. K. Ziabari, Z. Chen, R. Ubal, J. L. Abelln, J. Kim, A. Joshi, and D. Kaeli, “MG- PUSim: Enabling Multi-GPU Performance Modeling and Optimization,” in   Proceedings  of  the  46th  International   Symposium  on  Computer Architecture (ISCA), 2019, pp. 197–209.

[68]  J. Tuck, L. Ceze, and J. Torrellas, “Scalable Cache Miss Handling for High Memory-Level Parallelism,” in  Proceedings  of  the  39th Annual IEEE/ACM International  Symposium  on  Microarchitecture  (MICRO), 2006, pp. 409–422.

[69]  R. Ubal, B. Jang, P. Mistry, D. Schaa, and D. Kaeli, “Multi2Sim: A Simulation Framework for CPU-GPU Computing,” in Proceedings of the 21st International Conference on Parallel Architectures and Compilation Techniques (PACT), 2012, pp. 335–344.

[70]  O. Villa, M.  Stephenson, D. Nellans,  and  S. W.  Keckler,  “NVBit: A Dynamic  Binary  Instrumentation  Framework  for  NVIDIA  GPUs,”  in Proceedings of the 52Nd Annual IEEE/ACM International Symposium on Microarchitecture (MICRO), 2019, pp. 372–383.

[71]  X. Wang,  K.  Huang, A.  Knoll,  and  X.  Qian,  “A  Hybrid  Framework for Fast  and Accurate GPU Performance Estimation through  Source- Level  Analysis  and  Trace-Based  Simulation,”  in  IEEE  International Symposium on High Performance Computer Architecture (HPCA), 2019, pp. 506–518.

[72]  D.  Wong,  N.   S.   Kim,  and  M.  Annavaram,   “Approximating  Warps with Intra-warp Operand Value Similarity,” in 2016 IEEE International Symposium on High Performance Computer Architecture (HPCA), 2016, pp. 176–187.

[73]  H.    Wong,     M.-M.     Papadopoulou,    M.     Sadooghi-Alvandi,     and A.    Moshovos,     “Demystifying    GPU    Microarchitecture     through Microbenchmarking,” in IEEE International Symposium on Performance Analysis of Systems & Software (ISPASS), 2010, pp. 235–246.

[74]  G. Wu, J. L. Greathouse, A. Lyashevsky, N. Jayasena, and D. Chiou, “GPGPU Performance and Power Estimation Using Machine Learning,” in  IEEE 21st International Symposium on High Performance Computer Architecture (HPCA), 2015, pp. 564–576.

[75]  Y. Zhang and J. D. Owens, “A Quantitative Performance Analysis Model for GPU Architectures,” in  IEEE 17th International Symposium on High Performance Computer Architecture (HPCA), 2011, pp. 382–393.

[76]  H. Zhu, M. Akrout, B. Zheng, A. Pelegris, A. Jayarajan, A. Phanishayee, B.  Schroeder,  and  G.  Pekhimenko,  “Benchmarking  and  Analyzing Deep Neural Network Training,” in  IEEE International Symposium on Workload Characterization (IISWC), 2018, pp. 88–100.

486